#### Equipment

· Beakers or plastic containers
· A tea strainer or cheesecloth
· Measuring cylinder
· Stirring device (fork, tongue blade)
· Test tubes
· Test tube rack
· Methylene blue
· Microslide, coverslips
· Balance or teaspoon
· Microscope

#### Procedure

(a) Weigh or measure approximately 3 g of faeces into Container 1.

(b) Pour 40-50 ml of tap water into Container 1.

(c) Mix (stir) thoroughly with a stirring device (fork, tongue blade).

(d) Filter the faecal suspension through a tea strainer or double-layer of cheesecloth into Container 2.

(e) Pour the filtered material into a test tube.

(f) Allow to sediment for 5 minutes.

(g) Remove (pipette, decant) the supernatant very carefully.

(h) Resuspend the sediment in 5 ml of water.

(i) Allow to sediment for 5 minutes.

(j) Discard (pipette, decant) the supernatant very carefully.

(k) Stain the sediment by adding one drop of methylene blue.

(l) Transfer the sediment to a microslide. Cover with a coverslip.

Microscopical examination of prepared samples

The prepared samples on microslides from the simple test tube flotation method, the simple flotation method and the sedimentation method are examined under a microscope at different magnifications.

| Magnification | Parasites |
| --- | --- |
| 10 x 10 | Nematode and cestode eggs |
| 10 x 40 | Coccidia oocysts |
| 10 x 4 | Trematode eggs |